Quiz 1  Midterm     Computer Architecture    24  July 2000

1)  A computer has 2 instructions.

SUBS X	Substract the contents of location X from the accumulator, and store the result in the location X.

JUMP X  	Place address X in the program counter

Demonstrate that this instruction set is reasonably complete by specifying how the following operations can be programmed :

a) data transfer :  location X to/from accumulator

b) addition : add contents of location X to accumulator

c) conditional branching

d) logical OR

e) I/O operation

2)  Assume an instruction set that uses a fixed 16-bit instruction length.  Operand specifiers are 6 bits in length.  There are K two-operand instructions and L zero-operand instructions.  What is the maximum number of one-operand instructions that can be supported?

3)  Write the sequence of micro-operation (micro-step) for S1 hypothetical CPU to perform  the instruction  :  TWICE R1, R2    meaning double the value of the content

of R1 and store it to R2.

4)  Calculate CPI of a processor running the following benchmark :

Frequency of instruction executed

ADD R1, R2			3

ADD R3, R4			3

JP EQ :EXIT			taken 1/ not taken 2

MOV R1, R3			2

INC R1			2

JMP :LOOP			2

The number of clock for each instruction :

ADD  				2

JP  CONDITION    		taken 3 /  not taken 2

MOV  				2

INC    				2

JMP 				3